带物联网功能的太阳能控制器使用说明书

Solar controller instruction manual with Internet of Things function

—— CMC系列CMC series

本系列控制器具备太阳能板直接启动、电池激活功能，并内置物联网模块，可通过互联网远程客户端（比如Web/APP/等）查看和设置控制参数。可用于无人值守电源及应急电源需求的场合。

This series of controllers has direct solar panel activation, battery activation, and built-in IoT module to view and set control parameters via Internet remote clients (such as Web/APP/etc.). Can be used for unattended power and emergency power needs.

一、特点：Features:

●PV-Power功能：太阳能板直接启动控制器。

PV-Power function：solar panel can start controller directly.

●电池激活，电池维护功能，电池寿命更长久。

battery activation,battery protection,longer battery life.

●电池端0V电压或低压均能启动。

Battery side OV voltage or low voltage can be start.

●具有物联网通讯功能，可通过物联网模块实现控制器远程连接。

With IoT communication function, the controller can be remotely connected through the IoT module.

●采用GSM方式通信，内置物联卡。

Using GSM communication, Built-in IoT card.

●支持铅酸、三元锂电池、磷酸铁锂电池。

Support lead acid, ternary lithium battery, lithium iron phosphate battery.

●负载输出： 与电池同步。

Load output: Synchronize with battery.

●PC电脑端和手机APP可以监控CMC控制器的各种数据、控制和定时负载端的开关。

PC and mobile apps can monitor various data, control, and timing load switches of the CMC controller.

二、外观：Appearance

| 序号No. | 名称Name | 序号No. | 名称Name |
| --- | --- | --- | --- |
|  | 联网指示灯 Networking indicator | ⑥ | 光伏阵列接口 Photovoltaic array interface |
|  | 充电指示灯 Charging indicator light | ⑦ | 蓄电池接口 Battery interface |
|  | 电池电量指示灯 Battery indicator | ⑧ | 负载接口 Load interface |
|  | 负载输出指示灯 Load output indicator light | ⑨ | 开关/设置按键 Switch / setting button |
|  | 电池模式指示灯 Battery mode indicator light | ⑩ | 天线 Antenna |

三、设置说明：Setting instructions:

参数设置、保存与查看按键。短按此键是负载输出的开关键。长按此键5秒，电池模式指示灯闪动，表示已进入设置模式，设置完毕后，按一下5S保存退出；无操作10秒后保存退出。

Parameter setting, save and view button. Short press this key to activate the load output. Press this button for 5 seconds, battery mode indicator light flashing, indicating that the setting mode has been entered. After setting, press 5S to save and exit; save and exit after 10 seconds without operation.

四、参数：Parameter

| 型号Model | CMC-2410 | CMC-2420 |
| --- | --- | --- |
| 最大充电电流 Maximum charging current | 10A | 20A |
| 最大放电电流 Maximum discharge current | 10A | 20A |
| 最大光伏板耐压 Maximum photovoltaic panel withstand voltage | ≤50V |  |
| 特征feature | 空载损耗no-load loss：≤10mA；温度补偿Temperature compensation：-3mV/cell/℃ |  |
| 工作温度working Temp | -20℃～+60℃ |  |

| 电池类型battery type 参数Parameters | 额定电压nominal voltage | PV充电截止电压PV charge cut-off voltage | 电池放电截止电压Battery discharge cut-off voltage | 恢复放电电压Recovery discharge voltage |
| --- | --- | --- | --- | --- |
| 铅酸Lead-acid | 12V/24V | 14.4V/29V | 10V/21V | 12V/24V |
| 三元锂电池Ternary lithium battery*3串pcs | 3.7V*3 | 4.2V*3 | 3.0V*3 | 3.7V*3 |
| 磷酸铁锂电池Lithium iron phosphate battery*4串pcs | 3.2V*4 | 3.45V*4 | 2.5V*4 | 3V*4 |

※参数可根据用户要求定制。Parameters can be customized

五、物联网链接及使用 Internet link and using

- 方式一 way 1.

PC电脑网页: 打开http://solar.sunipcam.com

（可向厂家索取测试账号和密码）

（You can request a test account and password from the manufacturer）

手机网页: 打开http://solar.sunipcam.com

（注意：手机屏幕小，可以用手指向左滑动手机屏幕顶端选择项目）

（Note: The phone screen is small, and you can use your fingers to slide the top of the phone screen to the left to select items）

- 平台上的网页使用指示

Instructions for using web pages on the platform

（1）位置①是账号登录与切换。

（Position ① is account log in and switch）

（2）位置②是控制器的选择和负载的开关控制。

（Position ② is the selection of the controller and the switch control of the load）

（3）位置③是各种参数的记录表。

（Position ③ is a record table for various parameters）

（4）位置④是各种数据的更新按键。

（Position ④ is the update button for various data.）

（5）位置⑤是控制器分组和名字的更改。

（Location ⑤ is a change in controller grouping and name.）

（6）位置⑥是负载的定时开关的设置。

（Position ⑥ is the setting of the timing switch for the load.）

（7）位置⑦是设备添加分组。

（Location ⑦ is the device adding grouping.）

2、方式二 Way 2

①手机APP

用百度扫描二维码下载手机APP

Scan QR code on Baidu to download mobile app

②展示页面示例Display page example：