CPL Usage Instructions

一、Product Overview

- This series of controllers integrates solar Maximum Power Point Tracking (MPPT) charging technology. Compared to Pulse Width Modulation (PWM) designs, MPPT designs offer a 15% to 25% increase in charging efficiency. By charging at the PV's maximum power point, these controllers significantly enhance charging power and reduce system costs. This product boasts several notable features:

- ①High-Quality Components: Utilizes high-quality, long-lasting components to improve system performance and ensure a long service life.

- ②Efficient DC-DC Conversion: Achieves a maximum DC-DC conversion efficiency of up to 98% during MPPT charging.

- ③Advanced MPPT Technology: Employs advanced MPPT algorithms with a tracking efficiency of no less than 99.5%.

- ④Rapid MPPT Response: Features ultra-fast maximum power point tracking speed while maintaining tracking efficiency.

- ⑤IP65 Protection Rating: Equipped with an IP65 protection rating for dust tightness and water resistance.

- ⑥Comprehensive Protection: Offers complete protection functions against overcharging and other conditions, ensuring safety when the battery is fully charged.

- ⑦Temperature Compensation: Includes temperature compensation functionality to optimize performance under varying environmental conditions.

- ⑧Aluminum Alloy Heat Dissipation Housing: Features an aluminum alloy casing designed for excellent heat dissipation properties.

二、Product Appearance

| ① | Charging Indicator Light | ⑤ | Fine Tune Increase |
| --- | --- | --- | --- |
| ② | System Voltage | ⑥ | Fine Tune Decrease |
| ③ | Charging Cutoff Voltage | ⑦ | PV |
| ④ | System Voltage Switching | ⑧ | BAT |

三、User Manual

1、Normal Operating Status: The green light is on.

Charging Status: When the system enters the charging state, Indicator Light ① will turn red.

2、To Switch System Voltage: Press and hold System Voltage Setting Button ④ for three seconds to cycle to the next available system voltage setting.

3、To Enter Fine Adjustment Mode: Press and hold Fine Adjustment Button ⑤ or ⑥ for three seconds to enter Fine Adjustment Mode. In this mode, you can adjust the charging cutoff voltage.

四、Technical Specifications

| Model | CPL-7208 |  |  |  | CPL-7210 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Battery Type | Battery Type: Lead-Acid Battery with Custom Specifications |  |  |  |  |  |  |  |
| Rated Power | Battery Voltage*8A |  |  |  | Battery Voltage*10A |  |  |  |
| Maximum Charging Power | 651W |  |  |  | 814W |  |  |  |
| MPPT Tracking Efficiency | >99% |  |  |  |  |  |  |  |
| PV Terminal Voltage Rating | 150V |  |  |  |  |  |  |  |
| Temperature Compensation | -3mv/℃/2V |  |  |  |  |  |  |  |
| No-Load Current | ≤10mA |  |  |  |  |  |  |  |
| Operating Temperature Range | -25℃～+65℃ |  |  |  |  |  |  |  |
| System Voltage | 29.8V | 12V | 24V | 36V |  | 48V | 60V | 72V |
| Charging Cutoff | 29.8V | 14.5V | 29V | 43.5V |  | 58V | 70V | 85V |
| Charge Recovery | 29.8V | 13.9V | 27.8V | 41.7V |  | 55.6V | 67V | 81.4V |
| Fine Adjustment Range | 29.8V | 12.8V ~ 14.5V | 25.6V ~ 29V | 38.4V ~ 43.5V |  | 51.2V ~ 58V | 64V ~ 72.5V | 76.8V ~ 87V |
| IP Rating | IP65 |  |  |  |  |  |  |  |
| Dimensions (Length × Width × Height) | 100×103.8×28.3mm |  |  |  |  |  |  |  |
| Weight | 647G |  |  |  |  |  |  |  |